# MOD 19/95

## Changes on TOP432DIP

This change is required for the SST 28SF040 device.

The SST part requires one additional fast clock which is not connected on the TOP432DIP. To read, program or test these devices one additional connection is required.

A qualified Surface Mount Technician must do the required MOD(s).  If you do not have one available, you should contact the nearest Data I/O approved Service Center.  Please note that this is a billable service.

The changes are:

| - | Remove the TOP432DIP from the programmer |
| --- | --- |
| - | Turn the TOP432DIP so that the 32 pin sockets are on the bottom |
| - | Loosing the four screws and remove the plastic case of the TOP |
| - | Add one 10K Ohm resistorr between pin 21 and pin 28 of the single connector on the right side of the PCB board (see schematic) |
| - | Replace the plastic case onto the TOP and fasten the screws |
| - | Place the TOP432DIP onto the programmer |

Check your work by

| - | running the SPRINT selftest utility |
| --- | --- |
| - | running the PROM.EXE and select the device SST 28SF040 |