Document Version: 2001.001

Dear SPRINT PLUS48 user,

when you get the screen message "Please insert JPx and JPy" a hardware change is necessary for that special device you want to program. This modification is concerning some devices who have the supply-pins not at the standard locations. Look at the schematic below to locate the position of the jumpers on the PLUS48 PCB.

1.)	Disconnect the power- and the printerport cable from the PLUS48.

2.)	Remove the four screws (or the plastic parts) on the sides of the PLUS48 case.

3.)	Remove the bottom sheet and insert or remove the required jumpers.

4.)	Attach the PLUS48, connect both cables and start the Sprint software. The screen 
	message should no longer appear, if the jumpers are in the right positions.

- PLUS48 PCB jumper positions

Only one jumper should be set in each of the three available
jumper-blocks  : (J5 or J6 or J7) , (J2 or J3) , (J10 or J11) !!

| Device | Package | Adapter | Jumper-setting |
| --- | --- | --- | --- |
| PIC16C54/56 | all |  | J3, J7 |
| PIC16C71/84 | all |  | J3, J7 |
| PIC17C42 | DIL |  | J2 (default) |
| PIC17C42 | PLCC | S493 | J10 |
| Cypress 7C259 | PLCC | S493 | J2 (default) |
| Intel 87(C)51xx/ Philips 87(C)5x/ SiemensSAB5xx/ TEMIC87Cxxx | PLCC | S493 | J10 |
| Lattice (is)pLSI1016 | PLCC | S493 | J7 |